📘 Enterprise Network Design & Implementation Documentation

Course: MIT 565 – Internetworking
Student: Gaurishankar Bikram Jirel
Institution: Elmhurst University
Semester: Spring 2026

1. 🧭 Network Design Overview

This project implements a multi-branch enterprise network consisting of three geographically distributed locations: Chicago, New York, and Indiana. All branches are interconnected through a centralized core router (RouterHub), forming a hybrid routed network.

The design demonstrates interoperability between multiple dynamic routing protocols:

- Chicago and New York branches use OSPF

- Indiana branch uses EIGRP

To enable seamless communication across these distinct routing domains, RouterHub performs mutual route redistribution, ensuring full reachability between all networks.

2. 🌐 IP Addressing & VLAN Segmentation

Each branch is logically segmented using VLANs to improve scalability, traffic isolation, and security.

2.1 VLAN Allocation

Chicago Branch

- VLAN 10 (HR): 192.168.10.0/24

- VLAN 20: 192.168.20.0/24

- VLAN 30 (IT): 192.168.30.0/24

New York Branch

- VLAN 10: 192.168.12.0/24

- VLAN 20: 192.168.22.0/24

- VLAN 30: 192.168.32.0/24

Indiana Branch

- VLAN 10: 192.168.11.0/24

- VLAN 20: 192.168.21.0/24

- VLAN 30: 192.168.31.0/24

2.2 Inter-Router Links

Point-to-point WAN links use /30 subnetting for efficient IP utilization:

- Chicago ↔ RouterHub: 10.0.16.0/30

- Indiana ↔ RouterHub: 10.0.17.0/30

- New York ↔ RouterHub: 10.0.18.0/30

3. 🔁 Inter-VLAN Routing

Inter-VLAN communication is achieved using the Router-on-a-Stick approach.

- Routers are configured with subinterfaces for each VLAN.

- Each subinterface is associated with a VLAN using 802.1Q encapsulation.

- Default gateways for end devices are provided through these subinterfaces.

This design allows centralized Layer 3 routing while maintaining Layer 2 segmentation.

4. 📡 Dynamic Routing Configuration

4.1 OSPF Configuration (Chicago & New York)

- OSPF is used as the Interior Gateway Protocol for Chicago and New York branches.

- Routers form neighbor adjacencies and exchange routing information dynamically.

- Ensures fast convergence and scalable routing.

4.2 EIGRP Configuration (Indiana)

- Indiana branch uses EIGRP for routing.

- EIGRP provides efficient route calculation using composite metrics (bandwidth, delay, etc.).

4.3 Route Redistribution (RouterHub)

RouterHub integrates both routing domains through bidirectional redistribution:

- OSPF routes are redistributed into EIGRP

- EIGRP routes are redistributed into OSPF

EIGRP Metric Configuration

To ensure proper redistribution into EIGRP, the following metric is configured:

metric 10000 100 255 1 1500

This defines:

- Bandwidth: 10000 kbps

- Delay: 100 microseconds

- Reliability: 255

- Load: 1

- MTU: 1500

This configuration ensures accurate route propagation and prevents routing inconsistencies.

5. 🔐 Network Security Implementation

5.1 Access Control Lists (ACLs)

An extended ACL is implemented in the Chicago branch to enforce departmental isolation:

- Traffic from HR VLAN (VLAN 10) is denied access to IT VLAN (VLAN 30)

- Applied on the router interface connected to the VLAN network

This ensures sensitive HR data is protected from unauthorized access by other departments.

5.2 Port Security

Port security is configured on all user-facing access ports to prevent unauthorized device connections.

Key Features:

- Sticky MAC address learning enabled

- MAC addresses are dynamically learned and stored in the running configuration

- Maximum MAC addresses per port controlled

- Violation mode set to restrict

Behavior in Restrict Mode:

- Unauthorized MAC addresses are blocked

- Traffic from unknown devices is dropped

- Security violation is logged without shutting down the port

This enhances endpoint-level security while maintaining network availability.

6. 🔗 Layer 2 Technologies

6.1 Spanning Tree Protocol (STP)

STP is implemented to prevent Layer 2 loops and ensure a loop-free topology.

Additional Enhancements:

- PortFast enabled on access ports

  - Allows faster transition to forwarding state

- BPDU Guard enabled

  - Protects against rogue switches by disabling ports receiving unexpected BPDUs

These features improve both performance and security.

6.2 EtherChannel (Link Aggregation)

EtherChannel is configured between switches using:

- Link Aggregation Control Protocol (Active mode)

Benefits:

- Combines multiple physical links into a single logical link

- Provides redundancy and increased bandwidth

- Load balancing across links

7. 📡 DHCP Configuration

Dynamic IP address assignment is implemented using DHCP:

- Separate DHCP pools are configured for each VLAN in each branch

- Each pool provides:

  - Network address

  - Default gateway

  - DNS (if configured)

This simplifies IP management and reduces manual configuration errors.

8. ✅ Network Verification & Testing

Connectivity and configuration were verified using:

- Ping tests across VLANs and branches

- Routing table verification (show ip route)

- OSPF neighbor status (show ip ospf neighbor)

- EIGRP verification (show ip eigrp neighbors)

- Spanning Tree status (show spanning-tree)

- Port security status (show port-security)

Results confirm:

- Full inter-branch connectivity

- Proper enforcement of ACL restrictions

- Stable routing across OSPF and EIGRP domains

9. 🧾 Conclusion

This network design successfully demonstrates:

- Multi-protocol routing (OSPF + EIGRP)

- Route redistribution for interoperability

- VLAN-based segmentation

- Secure access control mechanisms

- Layer 2 optimization using STP and EtherChannel

- Scalable IP management using DHCP

The implementation reflects real-world enterprise networking practices, focusing on security, scalability, and reliability.